/*

* ORFA -- Open Robotics Firmware Architecture

*

* Copyright (c) 2009 Vladimir Ermakov, Andrey Demenev

*

* Permission is hereby granted, free of charge, to any person obtaining a copy

* of this software and associated documentation files (the "Software"), to deal

* in the Software without restriction, including without limitation the rights

* to use, copy, modify, merge, publish, distribute, sublicense, and/or sell

* copies of the Software, and to permit persons to whom the Software is

* furnished to do so, subject to the following conditions:

*

* The above copyright notice and this permission notice shall be included in

* all copies or substantial portions of the Software.

*

* THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR

* IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY,

* FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE

* AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER

* LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM,

* OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN

* THE SOFTWARE.

*****************************************************************************/

/** Servo 4017 low level driver

* @file servo/4017/servo_lld.c

*

* @author Andrey Demenev

*/

#include <avr/io.h>

#include <avr/interrupt.h>

#include <avr/pgmspace.h>

#include <util/atomic.h>

#include "servo_lld.h"

/// Debug print

#ifndef NDEBUG

#include <stdio.h>

#define debug(...) printf(__VA_ARGS__)

#else

#define debug(...)

#endif

#define US2CLOCK(us) (((uint32_t)(us) * (uint32_t)(F_CPU / 8000000.0 * 0x10000UL)) >>)

#define process_timer(OCRX, TCCRX, block, FOC_MASK) { \

OCRX += *(table_ptr[block]); \

if (table_ptr[block] == &(calc_ocr[block][8]) ) { \

table_ptr[block] = &(calc_ocr[block][0]); \

TCCRX |= FOC_MASK; \

TCCRX |= FOC_MASK; \

} else { \

table_ptr[block]++; \

} \

TCCRX |= FOC_MASK; \

}

static uint16_t servo_pos[32];

static uint8_t PROGMEM pin_map[32] = {

7, 3, 2, 6, 5, 1, 0, 4,

7, 3, 2, 6, 5, 1, 0, 4,

4, 0, 1, 5, 6, 2, 3, 7,

4, 0, 1, 5, 6, 2, 3, 7,

};

static uint16_t calc_ocr[4][9] = {

{

US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500),

US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500),

US2CLOCK(20500 - 8 * 1500),

},

{

US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500),

US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500),

US2CLOCK(20500 - 8 * 1500),

},

{

US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500),

US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500),

US2CLOCK(20500 - 8 * 1500),

},

{

US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500),

US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500), US2CLOCK(1500),

US2CLOCK(20500 - 8 * 1500),

},

};

static uint16_t* table_ptr[4] = {

calc_ocr[0],

calc_ocr[1],

calc_ocr[2],

calc_ocr[3],

};

ISR(SIG_OUTPUT_COMPARE3A) {

process_timer(OCR3A, TCCR3C, 2, (1 << FOC3A));

}

ISR(SIG_OUTPUT_COMPARE3B) {

process_timer(OCR3B, TCCR3C, 0, (1 << FOC3B));

}

ISR(SIG_OUTPUT_COMPARE3C) {

process_timer(OCR3C, TCCR3C, 3, (1 << FOC3C));

}

ISR(SIG_OUTPUT_COMPARE1C) {

process_timer(OCR1C, TCCR1C, 1, (1 << FOC1C));

}

uint16_t servo_lld_get_position(uint8_t n)

{

if (n > SERVO_CHMAX)

return 0;

return servo_pos[n];

}

void servo_lld_set_position(uint8_t n, uint16_t pos)

{

if (n > SERVO_CHMAX)

return;

debug("#sp %d %d\n", n, pos);

if (pos < 500)

pos = 500;

else if (pos > 2500)

pos = 2500;

uint8_t idx = pgm_read_byte(pin_map+n);

uint8_t block = n >> 3;

uint16_t uspos = US2CLOCK(pos);

debug("#s=%d p=%d\n", n, uspos);

uint16_t* pos_p = servo_pos+n;

uint16_t* pause_p = calc_ocr[block]+8;

uint16_t* pulse_p = calc_ocr[block]+idx;

uint16_t pause = *pause_p;

uint16_t pulse = *pulse_p;

pause += pulse;

pause -= uspos;

ATOMIC_BLOCK(ATOMIC_RESTORESTATE) {

*pause_p = pause;

*pulse_p = uspos;

*pos_p = pos;

}

}

void servo_lld_init(void)

{

DDRE = _BV(2) | _BV(3) | _BV(4) | _BV(5);

DDRB = _BV(7);

PORTE |= _BV(2);

OCR3A = 2000;

OCR3B = 2000;

OCR3C = 2000;

OCR1C = 2000;

TCCR3A = _BV(COM3A0) | _BV(COM3B0) | _BV(COM3C0);

TCCR3B = _BV(CS31);

TCCR1A = _BV(COM1C0);

TCCR1B = _BV(CS11);

ETIMSK |= _BV(OCIE1C) | _BV(OCIE3A) | _BV(OCIE3B) | _BV(OCIE3C);

PORTE &= ~_BV(2);

}